4XSTRUCT · STRUCTMIND AI

SUSTAINABILITY REPORT

Project: Quick Analysis

Generated: 2026-04-24T19:16:12.282331+00:00

Model: STRUCTMIND CORE · PRO

SHA-256 Hash: 12b93db7a65e57e7d5e82f64bc59ae929868415d9152989afde994357c0cb212

Of course. Here is the STRUCTMIND AI Sustainability Report for the specified structural steel package.

*

### STRUCTMIND AI Sustainability Report

This report analyzes the key sustainability metrics for the procurement and delivery of 220 tons of structural steel, comparing Electric Arc Furnace (EAF) and Basic Oxygen Furnace (BOF) production methods.

Project Summary:

| Parameter | Specification |
| --- | --- |
| Material | Structural Steel |
| Quantity | 220 tons |
| Manufacturing Process | EAF (Electric Arc Furnace) vs. BOF (Basic Oxygen Furnace) |
| Coating System | SSPC 3-Coat System (e.g., Zinc/Epoxy/Polyurethane) |
| Transportation | Truck, 900 miles (from mill/fabricator to site) |

---

### 1. Embodied Carbon Estimate (A1-A3, Cradle-to-Gate)

This estimate covers emissions from raw material extraction through the manufacturing process, before transportation. The choice of mill type is the single most significant factor.

| Mill Type | Embodied Carbon (per ton) | Total Embodied Carbon (220 tons) | Analysis |
| --- | --- | --- | --- |
| EAF Steel | ~600 kgCO₂e / ton | 132,000 kgCO₂e (132 metric tons) | Significantly lower carbon footprint due to the use of recycled scrap steel as the primary feedstock, powered by electricity. |
| BOF Steel | ~2,200 kgCO₂e / ton | 484,000 kgCO₂e (484 metric tons) | Substantially higher footprint due to the carbon-intensive process of converting iron ore into steel using metallurgical coal. |

Conclusion: Specifying steel from an EAF mill results in an approximate 74% reduction in cradle-to-gate embodied carbon for this project.

---

### 2. Recycled Content % Target

Recycled content is directly linked to the production method and is critical for circular economy goals and LEED credits.

EAF Mills Target: >93%

EAF production primarily melts scrap steel. Typical recycled content ranges from 93-98%, making it a cornerstone of steel recycling.

BOF Mills Target: 25-35%

BOF production is limited in the amount of scrap it can use. Recycled content is supplemental and significantly lower.

---

### 3. Transportation Carbon Estimate (A4, Transport-to-Site)

This calculation estimates the emissions from transporting the 220 tons of fabricated steel 900 miles by heavy-duty truck.

Total Ton-Miles: 220 tons × 900 miles = 198,000 ton-miles

Emission Factor (Truck): ~0.16 kg CO₂e per ton-mile

Calculation: 198,000 ton-miles × 0.16 kg CO₂e/ton-mile = 31,680 kg CO₂e

Estimated Transport Carbon: ~31.7 metric tons CO₂e

Recommendation: While this is a fixed parameter for this report, sourcing from a fabricator closer to the project site is a key strategy for reducing transport emissions.

---

### 4. Coating VOC / Low-Emission Options

A standard SSPC 3-coat system (e.g., inorganic zinc primer, epoxy mid-coat, polyurethane topcoat) provides excellent corrosion protection but can contain high levels of Volatile Organic Compounds (VOCs).

Standard System VOCs: Can range from 250 to 400 g/L. VOCs contribute to smog formation and poor air quality.

Low-Emission Recommendations:

Specify Low-VOC Formulations: Mandate coatings with VOC content <100 g/L. Most major manufacturers offer high-performance, low-VOC equivalents.

Consider High-Solids Coatings: These systems have less solvent to evaporate, reducing VOC emissions and often requiring fewer coats.

Shop Application: Prioritize coating application in a controlled shop environment rather than on-site to manage emissions and improve quality control.

---

### 5. Material Efficiency & Waste Factor

Structural steel fabrication is highly efficient due to advanced modeling and cutting technologies.

Target Waste Factor: < 5%

Process: Modern fabrication utilizes BIM models and nesting software to optimize cutting patterns from standard steel sections, minimizing off-cuts.

Circularity: All steel off-cuts and scrap generated during fabrication are 100% recyclable and are typically collected and sent back to an EAF mill, creating a closed-loop system.

---

### 6. EPD / LEED / BREEAM Contribution Summary

To claim sustainability credits, documentation is essential. Specifying steel from a mill that provides an Environmental Product Declaration (EPD) is the first and most critical step.

#### LEED v4.1 MRc Contribution (Focus on EAF Steel)

The 220 tons of EAF-produced steel can contribute significantly to the following Materials and Resources (MR) credits:

| LEED v4.1 Credit | Contribution Path & Analysis |
| --- | --- |
| MRc: Building Product Disclosure & Optimization – EPDs | (Potential: 1-2 points)<br>• Option 1 (1 point): Obtain a product-specific Type III EPD from the steel mill/fabricator. This is the most common and achievable path.<br>• Option 2 (2 points): If the EPD demonstrates an embodied carbon value below the industry average, an additional point for optimization can be achieved. EAF steel is inherently positioned to meet this. |
| MRc: Building Product Disclosure & Optimization – Sourcing of Raw Materials | (Potential: 1 point)<br>• Option 2 (Recycled Content): EAF steel with >93% post-consumer and pre-consumer recycled content is a major contributor. The material's cost is multiplied by its recycled content percentage to contribute to the project's overall threshold (typically 25% by cost). |
| MRc: Building Product Disclosure & Optimization – Material Ingredients | (Potential: 1 point)<br>• This credit is less commonly achieved with raw structural steel. However, if the fabricator can provide a Health Product Declaration (HPD) or Declare Label for the finished members, it can contribute. |

Summary for LEED: By specifying EAF steel with a product-specific EPD, this package can confidently contribute toward at least two LEED points under the Materials and Resources category.