4XSTRUCT · STRUCTMIND AI

SUSTAINABILITY REPORT

Project: Quick Analysis

Generated: 2026-04-24T19:11:38.543188+00:00

Model: STRUCTMIND CORE · PRO

SHA-256 Hash: 37327775ce5cba1423f3e011bca31d965c4c1911ce0133f184fd8c10e661f75f

Of course. Here is the STRUCTMIND AI Sustainability Report for the specified structural steel package.

*

### STRUCTMIND AI: Sustainability Report

Project Component: Structural Steel Package Total Tonnage: 220 tons (199.6 metric tons) Manufacturing Process: Comparative Analysis: Electric Arc Furnace (EAF) vs. Basic Oxygen Furnace (BOF) Coating System: Standard SSPC 3-Coat System (Assumed: Zinc/Epoxy/Urethane) Transportation: Truck Freight, 900 miles (1,450 km)

---

### 1. Embodied Carbon Estimate (A1-A3 Manufacturing)

This estimates the "cradle-to-gate" carbon emissions from raw material extraction through the manufacturing process. The choice of mill is the single most significant factor.

Electric Arc Furnace (EAF) Mills:

kgCO₂e per ton: ~600 kgCO₂e / ton (0.6 tCO₂e / ton)

Total Project Embodied Carbon: 132,000 kgCO₂e (132 metric tons CO₂e)

Basic Oxygen Furnace (BOF) Mills:

kgCO₂e per ton: ~2,200 kgCO₂e / ton (2.2 tCO₂e / ton)

Total Project Embodied Carbon: 484,000 kgCO₂e (484 metric tons CO₂e)

Analysis: Specifying steel from an EAF mill reduces the manufacturing-stage embodied carbon of this package by approximately 73% (a saving of 352 metric tons of CO₂e) compared to steel from a typical BOF mill. This is due to EAF's use of recycled scrap as its primary feedstock.

---

### 2. Recycled Content % Target

Recycled content is the primary driver of the embodied carbon difference.

EAF Mills: The feedstock is almost entirely recycled scrap steel.

Target Recycled Content: >90% (Typically 93-97%)

BOF Mills: The feedstock is primarily virgin iron ore, with a smaller amount of scrap used for process control.

Typical Recycled Content: 25-35%

Recommendation: To meet sustainability goals, mandate steel from EAF mills to maximize recycled content.

---

### 3. Transportation Carbon Estimate (A4 Transport)

This estimates the "gate-to-site" emissions from transporting the fabricated steel.

Methodology:

Total Ton-Miles: 220 tons * 900 miles = 198,000 ton-miles

Emission Factor: ~0.16 kgCO₂e per ton-mile (Typical for heavy-duty truck freight)

Calculation:

198,000 ton-miles * 0.16 kgCO₂e/ton-mile = 31,680 kgCO₂e

Total Transportation Carbon: ~32 metric tons CO₂e

Analysis: While significant, transportation emissions are secondary to manufacturing emissions. Sourcing from a fabricator/mill closer to the project site is the most effective way to reduce this impact.

---

### 4. Coating VOC / Low-Emission Options

A standard 3-coat system (e.g., inorganic zinc primer, epoxy mid-coat, polyurethane topcoat) provides excellent corrosion protection but can have a high Volatile Organic Compound (VOC) content.

Standard System Impact: Traditional solvent-borne coatings can have VOC levels from 250 to 450 g/L. VOCs are precursors to smog and can impact air quality.

Low-Emission Alternatives:

Low/Zero-VOC Coatings: Specify paint systems with VOC content <100 g/L. Many high-performance, water-based, and high-solids formulations are available.

Powder Coating: A zero-VOC option, though typically limited to smaller components due to oven size constraints.

Hot-Dip Galvanizing (HDG): An alternative corrosion protection system with extremely high durability and zero on-site VOC emissions. Its embodied carbon should be evaluated separately if considered.

Recommendation: Require submittals for coating systems to include VOC content data. Specify a maximum allowable VOC limit (e.g., <100 g/L) in the project documents.

---

### 5. Material Efficiency & Waste Factor

During fabrication, waste is generated from cutting members to size and from remnants (off-cuts).

Estimated Waste Factor: 8-12% is a typical range for structural steel fabrication.

Project Waste Estimate: 220 tons * 10% = 22 tons of scrap steel.

Circularity: A key advantage of steel is that this waste is not landfilled. It is 100% recyclable and has high economic value. The 22 tons of scrap will be collected and sold back to a steel mill, typically an EAF, directly re-entering the supply chain. This creates a highly effective circular economy loop.

Recommendation: Encourage early collaboration with the fabricator to optimize material usage through design detailing (e.g., minimizing cuts, using standard stock lengths) to reduce initial waste generation.

---

### 6. EPD / LEED / BREEAM Contribution Summary

This steel package can make a significant and direct contribution to green building certifications.

EPD (Environmental Product Declaration): Request a mill-specific, product-specific Type III EPD. This is the official documentation verifying the embodied carbon, recycled content, and other environmental impacts. Most major EAF mills in North America provide EPDs.

LEED v4.1 Contribution (MR Category: Materials and Resources):

MRc: Building Product Disclosure and Optimization – EPDs:

Option 1 (1 point): Achievable by sourcing from a manufacturer (the steel mill) that provides a product-specific EPD.

Option 2 (1 point): Potentially achievable. Requires the sourced EAF steel's EPD to demonstrate a Global Warming Potential (GWP) lower than the industry-average baseline. EAF steel is highly likely to meet this.

MRc: Building Product Disclosure and Optimization – Sourcing of Raw Materials:

Option 2 (1 point): Achievable. This credit rewards products meeting high recycled content thresholds. The >90% recycled content of EAF steel easily qualifies for this credit.

LEED Summary: By specifying and procuring EAF-produced structural steel with a qualifying EPD, this 220-ton package can directly contribute to achieving up to 3 points under the LEED v4.1 MR category.

Disclaimer:* The carbon factors and estimates provided are based on industry averages and established databases for preliminary analysis. Final values should be confirmed using project-specific Environmental Product Declarations (EPDs) from the selected steel mill and fabricator.