4XSTRUCT · STRUCTMIND AI

SUSTAINABILITY REPORT

Project: Quick Analysis

Generated: 2026-04-24T19:08:52.102128+00:00

Model: STRUCTMIND CORE · PRO

SHA-256 Hash: ca27bf292460c3ab5245091e35feeef504ad3734b62fe2ec03efcfaf6f19c896

Of course. Here is the STRUCTMIND AI sustainability report for the specified structural steel package.

*

### Sustainability Report: 220 Tons of Structural Steel

Project Scope: 220 tons of structural steel, fabricated and delivered 900 miles via truck, finished with an SSPC 3-coat paint system. This report analyzes the environmental impact and outlines strategies for mitigation and green building certification contribution.

---

#### 1. Embodied Carbon Estimate (Cradle-to-Gate)

The production method of steel is the single largest determinant of its embodied carbon. The comparison between Electric Arc Furnace (EAF) and Basic Oxygen Furnace (BOF) is critical.

| Production Method | Embodied Carbon (per ton) | Total for 220 Tons | Key Characteristic |
| --- | --- | --- | --- |
| Electric Arc Furnace (EAF) | 600 - 800 kgCO₂e | 132,000 - 176,000 kgCO₂e | Uses recycled steel scrap; powered by electricity. Strongly Recommended. |
| Basic Oxygen Furnace (BOF) | 2,000 - 2,400 kgCO₂e | 440,000 - 528,000 kgCO₂e | Uses virgin iron ore; energy-intensive process. |

Analysis: Specifying EAF-produced steel results in a carbon savings of approximately 308,000 to 352,000 kgCO₂e (308 to 352 metric tons) for this project. This is a reduction of over 70% compared to BOF steel.

Recommendation: Mandate the use of steel from EAF mills in the project specifications.

---

#### 2. Recycled Content % Target

Recycled content is directly linked to the production method and is a key metric for circular economy principles and LEED credits.

EAF Mills Target: >90%

EAF steel is produced by melting scrap steel. The recycled content (post-consumer + pre-consumer) is typically between 90-98%. This is the industry standard for sustainable steel design.

BOF Mills Target: 20-30%

BOF steel primarily uses virgin iron ore. A smaller amount of scrap steel is added as a coolant, resulting in significantly lower recycled content.

---

#### 3. Transportation Carbon Estimate (Fabricator-to-Site)

This calculation estimates the emissions from delivering the fabricated steel to the job site.

Distance: 900 miles

Tonnage: 220 tons

Emission Factor (Heavy-Duty Truck): ~165 gCO₂e per ton-mile

Calculation: 900 miles × 220 tons × 165 gCO₂e/ton-mile = 32,670,000 gCO₂e

Total Transportation Carbon: 32,670 kgCO₂e (or 32.7 metric tons of CO₂e)

Mitigation: While often fixed, sourcing from a fabricator closer to the project site is the most effective way to reduce this value.

---

#### 4. Coating VOC / Low-Emission Options

A standard SSPC 3-coat system (e.g., Zinc-Rich Primer, Epoxy Intermediate, Polyurethane Topcoat) can contain significant Volatile Organic Compounds (VOCs), which are regulated air pollutants.

Standard System VOCs: Can range from 250 g/L to over 400 g/L.

Low-Emission Recommendations:

Specify Low-VOC Coatings: Mandate that all shop- and field-applied coatings comply with stringent environmental standards, such as SCAQMD Rule 1113 (<100 g/L for most industrial coatings).

High-Solids Formulations: Specify high-solids epoxy and polyurethane coatings, which have less solvent and lower VOC content per unit of coverage.

Powder Coating: For smaller components or sections where feasible, powder coating is a zero-VOC alternative that offers excellent durability.

---

#### 5. Material Efficiency & Waste Factor

Efficient fabrication is key to minimizing waste and the associated environmental impact.

Typical Waste Factor: 3-5% for structural steel fabrication. For 220 tons, this equates to 6.6 - 11 tons of scrap.

Optimization Strategies:

BIM Integration: Utilize the Building Information Model (BIM) for direct-to-fabrication outputs, minimizing errors and optimizing material cuts.

Nesting Software: Require the fabricator to use advanced nesting software to lay out cuts on raw steel sections, maximizing yield and minimizing off-cuts.

Design for Standard Lengths: Where possible, design members to utilize standard mill-produced lengths to reduce cutoff waste.

Circularity: A major advantage of steel is that 100% of the fabrication scrap is valuable and can be collected and recycled back into the EAF steelmaking process.

---

#### 6. EPD / LEED / BREEAM Contribution Summary

By specifying EAF steel and requiring proper documentation, this project can make a significant contribution to green building certifications like LEED v4.1.

Action Item: Require a product-specific, Type III Environmental Product Declaration (EPD) from the steel mill/supplier.

LEED v4.1 MR Credit Contribution:

MRc: Building Life-Cycle Impact Reduction (Option 4, Whole-Building LCA):

MAJOR CONTRIBUTION. Using EAF steel with its low embodied carbon value will dramatically reduce the project's Global Warming Potential (GWP) in a whole-building LCA, making it much easier to achieve the required 10% reduction compared to a baseline (which would assume industry-average or BOF steel).

MRc: Building Product Disclosure and Optimization - EPDs (Option 1):

CONTRIBUTES 1 PRODUCT. Providing a product-specific EPD for the 220 tons of structural steel contributes one product towards the 20 needed for this credit.

MRc: Building Product Disclosure and Optimization - Sourcing of Raw Materials (Option 2):

STRONG CONTRIBUTION. EAF steel with >90% recycled content (mostly post-consumer) will significantly contribute to this credit. The value of the steel package is multiplied by its recycled content percentage, helping the project meet the threshold of 25% (by cost) of total building materials.

EQc: Low-Emitting Materials:

Specifying and documenting the use of low-VOC coatings for all shop and field applications will contribute to earning this credit.

Summary: The strategic specification of EAF-produced structural steel, supported by an EPD and paired with low-VOC coatings, is one of the most impactful decisions for improving the sustainability profile of the building structure and maximizing contributions to LEED Materials & Resources (MR) credits.